Engineering  Circular No. 82

| No.ENG/misc-29(11)/2006 | Dated:01.05.2007 |
| --- | --- |
| Subject : Boiler Survey |  |
| As per the Rule 73(d) of Merchant Shipping (Cargo Ship Safety Construction and Survey) Rules, 1991, for all vessels more than 8 years old, an internal & external examination of the boiler is required to be carried out at intervals of one year. Auxiliary water-tube boilers which the Principal Officer is satisfied are being given correct feed water treatment with proper boiler water analysis shall be examined internally and externally at intervals not exceeding two and half years. The Directorate after carefully examining the request from the INSA, Classification Rules and Draft M.S. (Cargo Ship Construction and Survey) Rules and in order to ensure uniformity with the rules of the Classification Societies, henceforth following periodicity of boiler survey is applicable: (a) auxiliary water-tube boilers shall be examined internally and externally at intervals not exceeding two and half years subject to conditions specified in Note. (b) all other boilers including exhaust-gas boilers, super-heaters, economizers and domestic boilers (other than domestic boilers having heating surface of not more than five square meters and a working pressure of not more than 3.5 bar gauge) shall be examined internally and externally at intervals not exceeding two and half years Sr.No.; Survey; Survey interval in years. 1; Main boilers; 2 1/2 *. 2; Auxiliary boilers; 2 1/2 *. 3; Exhaust gas steam generators and economisers; 2 1/2 *. * Note : At least 2 surveys are to be carried out within any 5 years but interval between two consecutive surveys is not to exceed 3 years. The Recognized Organization during the Annual Survey shall review the following records since the last Boiler Survey: - Operation - Maintenance - Repair history - Feed water chemistry and submit report of the above to the concerned Principal Officer and incase of any adverse report same shall be brought immediately to the notice of the Directorate and Principal Officer. This circular is issued in supersession of the Engineering Circular No.77 issued earlier on the subject. This comes into effect from the date of issue of this circular and is issued with the approval of the Director General of Shipping. Sd/- ( D. Mehrotra ) Dy.Chief Surveyor-cum-Sr.DDG (Tech) Back to ENGG circulars main page |