Engine Circular No. 29

| NO: ENG/MMAM-37(6)/99 | Dated May 28, 2004 |
| --- | --- |
| Sub: Fire Extinguishing system for deep-fat cooking equipment |  |
| Sir, Please find enclosed herewith the Engineering Circular No. 29 for Fire Extinguishing system for deep-fat cooking equipment, for your information and necessary action. This may please be circulated to all concerned. The circular is also available on the website of the Directorate : www.dgshipping.com This issues with the approval of the Chief Surveyor with the Government of India Yours faithfully, Sd/- (D.Mehrotra) Engineer and Ship Surveyor |  |
| Engineering Circular No. 29 File No. : ENG/MMAM-37(6)/99 Dated : 28.05.2004. Subject :- Fire Extinguishing system for deep-fat cooking equipment As per SOLAS Amendments, 2000, CH.II-2 Regulation 10.6.4 deep-fat cooking equipment shall be fitted with the following : .1 an automatic or manual fire-extinguishing system approved by the Directorate. .2 a primary and backup thermostat with an alarm to alert the operator in the event of failure of either thermostat; .3 arrangements for automatically shutting of the electrical power upon activation of the fire-extinguishing system; .4 an alarm for indicating operation of the fire-extinguishing system in the galley where the equipment is installed; and .5 controls for manual operation of the fire-extinguishing system, which are clearly labeled for ready use by the crew. 2. Applicability : 1. Ships constructed on or after 1st July 2002. 2. For new installation of deep-fat cooking equipment for ships constructed before1st July 2002. 3. Type Approval : The D.G. Shipping shall consider type approval of the fire extinguishing system for deep fat cooking equipment, which are approved by other Maritime Administration / Classification Societies on behalf of the Maritime Administration or manufactured in India and undergone all tests as per the specification laid down by IMO, and approved by this Directorate. |  |
| Back to ENGG circulars main page |  |