DRAFT

MINISTRY OF PORTS, SHIPPING, AND WATERWAYS

(Shipping Wing)

New Delhi, the _____________,2026

Merchant Shipping (Carriage of Cargoes and Oil Fuels) Rules, 2026

G.S.R.------- (E)-In exercise of the powers conferred by Section ------  of Merchant Shipping Act, 2025 and in supersession of the Merchant Shipping (Carriage of Cargo) Rules, 1995, except as respects things done or omitted to be done before such supersession, the Central Government hereby makes the following rules namely:

PART I: General Provisions

- Short title, commencement:

(1) These rules may be called Merchant Shipping (Carriage of Cargoes, and Oil Fuels) Rules, 2026.

(2) They shall come into force on the date of their publication in the Official Gazette.

- Application: unless specified otherwise, these Rules shall apply to:

(1) Carriage of cargoes (except liquids in bulk, gases in bulk, and those aspects of carriage covered in other Rules) which owing to their particular hazards to ships or person on board, require special precautions to be taken.

(2) All Indian ships carrying or about to carry cargoes specified in these rules anywhere; and

(3) All ships carrying or about to carry cargoes specified in these rules in port or places in India,

- Exceptions: As provisioned in sub-section (a) of Section 114 of the Merchant Shipping Act, 2025, and unless otherwise specified, these rules shall not apply to the following:

(1) Ship's stores and security equipment as permitted by the Central Government for use of security personnel, in limited quantities; and

(2) Ship’s salvage equipment for the purpose of salvage services; and

(3) Indian ships of below 500GT, provided competent authority by notification or by specific or general order or notice or circular in writing may decide to the extend or permit other effective measures to ensure reasonable and equivalent safety of such ships taking into account the nature and conditions of sheltered voyage.

- Definition: In these rules, unless the context otherwise requires:

(1) "Act" means the Merchant Shipping Act, 2025.

(2)  The terms "Administration" or "Contracting Government" when used in International Conventions or Codes adopted by the International Maritime Organization, mean the "Central Government" in exercise of powers under the Act administering through Director General of Maritime Administration.

(3) “Approved” means:

(a) in respect of cargo stowage plans, equipment, packaging, cargo securing manuals, fumigation, identification of hazards of cargoes not listed in the relevant codes and related matters approved by Nautical Advisor to the Government of India or Principal Officer, regional Mercantile Marine Department or Deputy Nautical Advisor to the Government of India or Recognized Organization (RO) notified by the Central Government.

(b) in respect of construction and equipment under MARPOL 73/78, IBC and IGC Codes means approved by Chief Surveyor with the Government of India or Deputy Chief Surveyor with the Government of India or Recognized Organization (RO) notified by the Central Government.

(c) in respect of stability and strength of ship means approved by Chief Ship Surveyor with the Government of India or Deputy Chief Ship Surveyor with the Government of India or Recognized Organization (RO) notified by the Central Government.

(4) “Approved person” means any person approved by Rule 4(3) possessing the knowledge and experience required to conduct various tests on machinery, electrical installations, ships hull and structure, equipment, required under these rules in course of construction, major conversions/repairs, operations and maintenance of the ship and appointed under Section 8 of the Act.

(5) IMSBC Code" means the International Maritime Solid Bulk Cargoes Code, adopted by the International Maritime Organization by Resolution MSC.268(85), as may be amended.

(6) "BLU code" means The Code of Practice for the Safe Loading and Unloading of Bulk Carriers including BLU Manual adopted by the International Maritime Organization by Resolution A.862(20), as amended from time to time; incorporated as Supplement to the IMSBC Code.

(7) "Cargo Unit" means a vehicle, container, flat, pallet portable tank, packaged unit, or any other entity etc., and loading equipment or part thereof which belongs to the ship but is not fixed to the ship.

(8) "Cargo Stowage and Securing Code" or "CSS" means the Code of Safe Practice for Cargo Stowage and Securing adopted by the International Maritime Organization by Resolution A.714 (16), as amended.

(9) "CTU Code" means the 2014 Code of Practice for Packing of Cargo Transport Units, which is endorsed by IMO, ILO, and UNECE, and is as amended from time to time.

(10) “Cargo Transport Unit” is a freight container, swap body, vehicle, railway wagon or any other similar unit in particular when used in intermodal transport.

(11) “Competent Authority” means Nautical Adviser to the Government of India, for the purpose of these rules to supplement the provisions to ensure that appropriate information on cargo, its stowage and securing is provided along with specific precautions, where necessary, for the safe carriage of such cargoes.

(12) “Container Safety Convention” (CSC) means the Convention on Safe Containers,1972, adopted by the International Maritime Organization, as amended from time to time.

(13) “Director General” means the Director General of Maritime Administration appointed under Section 7(1) of the Act for the purpose of implementing and enforcing the provisions of these rules.

(14) "International Grain Code" or "Grain Code" means the International Code for the Safe Carriage of Grain in Bulk, adopted by the International Maritime Organization by Resolution MSC.23(59), as amended.

(15) The term “Grain” includes wheat, maize (corn), oats, rye, barley, rice, pulses, seeds, and processed forms thereof, whose behaviour is similar to that of grain in its natural state.

(16) "MARPOL 73/78 Convention" means the International Convention for Prevention of Pollution from Ships, in force, including its protocol adopted by the International Maritime Organization, as amended from time to time.

(17) Term "notice" includes a Merchant Shipping (MS) Notice, Directorate General of Maritime Administration Order or Departmental Circular, issued from time to time.

## (18) "Organization" means the International Maritime Organization (IMO) – a United Nation's specialized agency with responsibility for the safety and security of shipping and the prevention of marine pollution by ships.

(19) “Pour” means quantity of cargo poured through one hatch opening as one step in the loading plan i.e. from the time the spout is positioned over hatch opening until it is moved to another hatch opening.

(20) "Recommendation of the Organization" or "Recommended by Organization" includes its assembly or its committee resolution or its circular, as adopted by the Organization.

(21) "SOLAS Convention" or "SOLAS" means the International Convention for Safety of Life at Sea, 1974, in force including its protocols adopted by the International Maritime Organization and as amended from time to time.

(22) “Solid Bulk Cargo” means any cargo, other than liquid or gas, consisting of a combination of particles, granules or any larger pieces of material generally uniform in composition, which is loaded directly into the cargo spaces of a ship without any intermediate form of containment.

(23) “Terminal Representative” means a person appointed by the terminal or other facility where the ship is loading or unloading, who has responsibility for operations conducted by that terminal or facility with regard to the particular ship.

(24) "Timber Code" means the Code of Safe Practice for Ships Carrying Timber Deck Cargoes adopted by the International Maritime Organization by Resolution A.1048(27), as amended from time to time

(25) "Timber deck cargo" means a cargo of timber carried on an uncovered part of a freeboard or superstructure deck

(26) Meaning of other terms and abbreviations: All other terms and abbreviations which are used in these rules and defined in the Act, SOLAS 1974, MARPOL 73/78, CSC 1972 and applicable Codes for the purpose of safe carriage of cargoes, cargo units and cargo transport units shall have the meanings as defined in the Act and those applicable Regulations, Codes and Conventions.

- Requirements for the carriage of solid bulk cargoes other than grain: The carriage of solid bulk cargoes other than grain shall be in compliance with the relevant provisions of the IMSBC Code.

- Cargo Information:

(1) The ship owner, ship master of all ships loading any cargo, shall ensure that cargo information is made available to the ship by the shipper before the commencement of cargo operations;

(2) The shipper shall provide the master or his representative with appropriate information on the cargo sufficiently in advance of loading to enable the precautions which may be necessary for proper stowage and safe carriage of the cargo to be put into effect. Such information, in a format, prescribed by applicable recommendation of the Organization, such information shall be confirmed in writing and by appropriate shipping documents prior to loading the cargo on the ship;

(3) The cargo information shall include:

(a) in the case of general cargo, and of cargo carried in cargo units, a general description of the cargo, the gross mass of the cargo or of the cargo units, and any relevant special properties of the cargo. For the purpose of this Rule the cargo information required in sub-chapter 1.9 of the Code of Safe Practice for Cargo Stowage and Securing, adopted by the Organization by resolution A.714(17), as may be amended, shall be provided.

(b) in the case of a bulk cargo, information as required by Section 4 of the IMSBC Code shall be provided.

(c) in case of a bulk cargo not listed in IMSBC Code, the provisions of Section 1.3 of the IMSBC Code shall apply.

(4) Prior to loading cargo units on board ships, the Shipper shall ensure that the gross mass of such units is in accordance with gross mass declared on the shipping documents and ascertained in accordance with relevant notice(s) issued by the competent authority.

#### (5) In the case of cargo carried in a container, except for containers carried on a chassis or a trailer when such containers are driven on or off a ro-ro ship engaged in short international voyages as defined in SOLAS Regulation III/3, the gross mass according to Rule 6(4) above, shall be verified by the shipper, either by:

##### (a) weighing the packed container using calibrated and certified equipment; or

##### (b) weighing all packages and cargo items, including the mass of pallets, dunnage and other securing material to be packed in the container and adding the tare mass of the container to the sum of the single masses, applicable at the premises, where packing of the container was completed, using an approved & certified method as prescribed by the notice(s) issued from time to time.

##### (6) The shipper of a container shall ensure the verified gross mass, as recommended by Organization, is stated in the shipping document. The shipping document shall be:

##### (a) signed by a person duly authorized by the shipper; and

##### (b) submitted to the master or his representative and to the terminal representative sufficiently in advance, as required by the master or his representative, to be used in the preparation of the ship stowage plan.

#### (7) If the shipping document, with regard to a packed container, does not provide the verified gross mass and the master or his representative and the terminal representative have not obtained the verified gross mass of the packed container, it shall not be loaded on to the ship.

- Oxygen Analysis and Gas Detection Equipment:

(1) Every ship, when transporting a solid bulk cargo which is liable to emit a toxic or flammable gas or cause oxygen depletion in the cargo space, an appropriate instrument for measuring the concentration of gas and/or oxygen in the air, along with detailed instruction for use shall be provided.

(2) For every such gas instrument provided following needs to be complied with:

(a) Crew members on such ship, shall be trained in the use of such instrument(s) within two weeks of their joining or prior entering 'enclosed or confined spaces' or commencement of cargo operations which affect atmospheric conditions, whichever is earlier and same shall be recorded.

(b) Each such instrument shall be maintained and periodically calibrated as required by the manufacturer.

- Use of Pesticides in Ships:

(1) Every master of a ship shall take appropriate precautions in use of pesticides on board his ship, in particular for the purposes of fumigation in accordance with the concerned recommendation(s) developed by the Organization to ensure that life of his crew and persons involved in fumigation is not endangered.

(2) In-transit fumigation should only be carried out based upon recommendations of the Organization and in compliance with the notices issued by competent authority as defined in Rule 4(11), if any.

- Stowage and Securing: When stowing and securing any cargo on board ship, the Master shall comply with the provisions specified in the relevant Code for acceptability, stowage, and securing of cargoes.

(1) Cargo, cargo units and cargo transport units carried on or under deck shall be so loaded, stowed and secured, as to prevent as far as practicable, throughout the voyage, damage or hazard to the ship and the persons on board and loss of cargo overboard

(2) Cargo, cargo units and cargo transport units shall be so packed and secured within the unit as to prevent, throughout the voyage, damage or hazard to the ship and the persons on board

(3) Appropriate precautions shall be taken during loading and transport of heavy cargoes or cargoes with abnormal physical dimensions to ensure that no structural damage to the ship occurs and to maintain adequate stability throughout the voyage

(4) Appropriate precautions shall be taken during loading and transport of cargo units and cargo transport units on board ro-ro ships, especially with regard to the securing arrangements on board such ships and on the cargo units and cargo transport units and with regard to the strength of the securing points and lashings

(5) Freight containers shall not loaded to more than the maximum gross weight indicated on the Safety Approval Plate fixed thereon under the CSC as amended

(6) All cargoes, other than solid and liquid bulk cargoes, cargo units and cargo transport units, shall be loaded, stowed and secured on board ship throughout the voyage in accordance with the approved Cargo Securing Manual. In ships with ro-ro spaces as defined in SOLAS Regulation II-2/3.41, all securing of such cargoes, cargo units and cargo transport units in accordance with the approved Cargo Securing Manual shall be completed before the ship leaves the berth.

(8) Cargo Securing Manual specified in this Rule, shall be drawn up to a standard not less than relevant recommendation of the Organization.

- Material Safety Data Sheets: Ships carrying oil or oil fuel, as defined in regulation 1 of Annex I of MARPOL, shall be provided with Material Safety Data Sheets (MSDS), based on the relevant recommendations developed by the Organization, prior to loading such oil as cargo in bulk or bunkering of oil fuel.

- Prohibition of the blending of bulk liquid cargoes and production processes during sea voyages:

(1) The physical blending of bulk liquid cargoes during sea voyages is prohibited. Physical blending refers to the process whereby the ship's cargo pumps and pipelines are used to internally circulate two or more different cargoes with the intent to achieve a cargo with a new product designation. This prohibition does not preclude the master from undertaking cargo transfers for the safety of the ship or protection of the marine environment.

(2) the prohibition specified in the Rule 11(1) above, does not apply to the blending of products for use in the search and exploitation of seabed mineral resources on board ships used to facilitate such operations.

(3) any production process on board a ship during sea voyages is prohibited. Production processes refer to any deliberate operation whereby a chemical reaction between a ship's cargo and any other substance or cargo takes place.

(4) The prohibition specified in sub-rule 11(3) above, does not apply to the production processes of the cargoes for use in the search and exploitation of seabed mineral resources on board ships used to facilitate such operations.

Part II: Special Provisions for Solid Bulk Cargoes (other than grain):

- Acceptability for Shipment:

(1) Prior to loading a solid bulk cargo, the master shall be in possession of approved comprehensive information on the ships stability and the distribution of cargo for the standard loading conditions.

(2) The method of providing such information shall:

(a) conform to the requirements prescribed by the applicable ‘notice’ issued by the competent authority; and

(b) conform to SOLAS Regulation II-1/5-1; and Guidelines for the preparation of intact stability information (MSC/Circ.456); and the Revised guidance to the master for avoiding dangerous situations in adverse weather and sea conditions (MSC.1/Circ.1228); and

(c) approved loading instrument is accepted as a supplement to stability information provided in hard copy.

(2) Concentrates or other cargoes which may liquefy shall only be accepted by the master of the ship for loading when the actual moisture content of the cargo is less than its transportable moisture limit (TML). Notwithstanding this sub-rule, cargoes having moisture content in excess of the TML may be carried on a specially constructed or fitted cargo ship for confining cargo shift as approved by the central government for Indian ships and in case of foreign flag ship it is to be approved by the respective flag administration.

(3) Shipper he amendments include, amongst others that when a concentrate or other cargo which may liquefy is carried, the shipper shall provide the ship's Master with a signed certificate of the TML and declaration of the Moisture Content issued by an entity recognized by the Competent Authority.

(4) The master shall take special precautions for safe loading and carriage of bulk cargoes which possess chemical properties that may create a potential hazard and are not covered by the IMDG Code.

(5) The information in respect of sold bulk cargo as required by Rule 6(3)(b), amongst others, in the case of concentrates or other cargo that may liquefy, shall include information in the form of signed certificate on the Moisture Content and Transportable Moisture Limit of the cargo, issued by entity / laboratory approved by the Directorate General of Maritime Administration

- Carriage requirement:

(1) Every ship loading solid bulk cargoes other than grain shall comply with the requirements of the IMSBC Code and loading & unloading procedures stipulated in the BLU Code and BLU Manual of this code applicable to the type of cargo being carried.

(2) The master shall ensure that bulk cargoes shall be loaded and trimmed properly to reasonable level, as necessary, to the boundaries of the cargo space as to minimize the risk of shifting and maintain adequate stability throughout the intended voyage.

(3) When bulk cargoes are carried in "Tween-decks" of general cargo ships, the hatchways of such spaces shall be closed for cases where the loading information indicates an unacceptable level of stress on the bottom structure if the hatchways are left open.  The cargo shall be trimmed at reasonable level and either extend from side to side or secured by additional longitudinal divisions of sufficient strength. The safe load-carrying capacity of the "twin-decks" shall be observed to ensure that the deck-structure is not over stressed or overloaded. The Master shall be guided by the relevant sections of the code in respect of cargoes carried on tween Decker ship or general cargo ships.

(4) Every cargo ship of 500 gross tonnage and over constructed on or after 1984 and cargo ship of less than 500 gross tonnage constructed on or after 1st February, 1992, when carrying dangerous goods in solid form in bulk, except class 6.2 & class 7 of IMDG code, shall comply with SOLAS Regulation II-2/19, and thereafter be issued with a Document of Compliance with Special Requirements for Ships Carrying Dangerous Goods as per SOLAS Regulation II-2/19.4 in format as specified in First Schedule.

- Loading, unloading and stowage:

(1) The shipper, ship owner, ship master and the ports involved in the safe loading and unloading of solid bulk cargoes listed in the IMSBC Code shall strictly comply with the relevant responsibilities, duties and authority specified in the BLU Code and detailed procedures in the BLU Manual.

(2) The cargoes that are not listed or detailed information is not available, the shipper, ship owner, ship master and the concerned ports shall approach the competent authority as specified in Rule 4(11) without any delay.

(3) To enable the master to prevent excessive stresses in the ship structure, all ship engaged in shipment of such cargoes shall be provided with a booklet, duly approved by the Recognized Organization (RO), written in a language with which the ships Master and Officers responsible for cargo operations are familiar with.  If this language is not in English, the ship shall be provided with the booklet written also in the English language.  The booklet shall as a minimum include:

(a) stability data, as required by SOLAS Regulation II-1/5-1;

(b) ballasting and de-ballasting rates and capacities;

(c) maximum allowable load per unit surface area of the tank top plating;

(d) maximum allowable load per hold;

(e) general loading and unloading instructions with regard to the strength of the ships structure including any limitations on the most adverse operating conditions during loading, unloading, ballasting operation and the voyage;

(f) any special restrictions such as limitations on the most adverse operating conditions impost by the Central Government issued by a relevant notice, if any;

(g) where strength calculations are required maximum permissible forces and movement on the ship's hull during loading, unloading and the voyage.

(4) Before a solid bulk cargo is loaded or unloaded, the master and the terminal representative shall agree on a plan, drawn up in accordance with BLU code, which ensures that the permissible forces and moments on the ship are not exceeded during loading or unloading, and shall include the sequence, quantity and rate of loading or unloading, taking into consideration the speed of loading or unloading, the number of pours and de-ballasting or ballasting capability of the ship. The plan and any subsequent amendments thereto shall be lodged with the nearest authority of the port State. For this purpose, on the coast of India, the nearest Mercantile Marine Department (MMD) shall be treated as Port State Authority.

(5) The Master and terminal Representative shall ensure that loading and unloading operations are conducted in accordance with the agreed plan.

(6) If during loading or unloading any of the limits specified in the cargo plan as mentioned in Rule 14(4) are exceeded or likely to become so if the loading or unloading continues, the master has the right to suspend operation and has the obligation to notify accordingly to the authority specified in Rule 14(4) with which the aforesaid plan has been lodged. The master and the terminal representative shall ensure that corrective action is taken.  When unloading cargo, the master and terminal representative shall ensure that unloading method does not damage the ship’s structure.

(7) If any damage occurs, that could impair the structural capability or the watertight integrity or the ships essential engineering systems, then the damage is to be reported to the Recognized Organization (RO) or the Authority of the Port State in case of outside India.

(8) The master shall ensure that ships personnel continuously monitor cargo operations. Where possible, the ships draft shall be checked regularly during loading or unloading to confirm the tonnage figure supplied.  Each draft and tonnage observation shall be recorded in a cargo log-book or other equivalent ship’s log book.  If significant deviations from the agreed plan are detected, cargo or ballast operations or both shall be adjusted to ensure that the deviations are corrected.

(9) The competent authority as specified in Rule 4(11) in consultation with ship owners, ports, shippers and other entities may issue notice detailing procedures and guidelines, from time to time, to implement and enforce the provisions of IMSBC Code that includes BLU Code and BLU Manual effectively.

Part III: Carriage of cargoes other than solid bulk cargoes:

- Carriage of grain:

(1) In addition to any other applicable requirements under the Rules, a cargo ship carrying grain shall comply with the requirement of the International Grain Code and hold a document of authorization approved by the Central Government.

(2) For a ship loading grain, the Master of the ship shall satisfy to the Principal Officer, of the Mercantile Marine Department of the district or his authorized official that the ship complies with the requirements of these Rules and the provisions of International Code for Safe Carriage of Grain in Bulk (Resolution MSC.23(59)) in its proposed loaded condition.

(3) A ship without a document required under Rule 15(1), shall not load grain until the master demonstrates to the satisfaction of competent authority as defined in Rule 4(11), or of the Contracting Government of the port of loading, that the ship in its loaded condition the ship shall comply with the will comply with the provisions of International Code for Safe Carriage of Grain in Bulk (Resolution MSC.23(59)).

- Carriage of timber cargoes:

(1) Every ship carrying timber deck cargo and timber inside the hold shall comply with the provisions of the recommendation issued by the Organization, amended and adopted from time to time.

(2) Ships that are provided with and making use of their timber load line should also comply with the requirements of the applicable regulations of the International Load Line Convention, as amended, and shall load timber under-deck and deck cargo as per approved stability booklet conditions.

(3) A vessel loaded with timber deck cargo, shall demonstrate compliance with Rule 16(1) and 16(2) to the Principal Officer, of the Mercantile Marine Department of the district or his authorized or concerned Port State Control officer if outside India.

- Carriage of Cargo Transport Units (containers)

(1) The Master of a ship shall ensures that new or existing containers loaded, stowed and carried for sea transportation have a safety approval plate conforming to the specifications set out in the appendix of the CSC, 1972.

(2) The Master of the ship shall ensure that every such container shipped on board has been provided with shipping documents prescribed in Rules 6 and 9, relevant to such container.

(3) It is the responsibility of the owner or Master of the ship to ensure that the compliance with the provisions of IMDG Code for the dangerous goods carried in the containers are verified.

(4) The Master or the owner shall ensure that containers received on board are well maintained and relevant endorsement is highlighted on the safety approval plate.

(5) The master of a ship when loading Cargo Transport Units (CTU) on his ship, shall ensure that shipper provides documents showing compliance with the Code of Practice for the Packing of Cargo Transport Units with special reference to its packing.

- Carriage of cargo on vessels registered under Part XIII of the Act:

(1) An Indian vessel registered under provisions of Part XIII of the Act, anywhere and any other non-conventional vessel of wooden construction, when in Indian waters shall not carry following cargoes-

- Any cargo in bulk, which comes in direct contact with her wooden hull and does not allow access to repair/attend to wooden hull, as may be necessary in need; or

- Dangerous cargoes.

(2) For purpose of promoting coastal trade, in India and with neighboring littoral countries and for providing such balance of trade between neighboring littoral countries or for supporting any other Government of India policy/initiative, the Director General may prescribe carriage of certain type of cargoes on board Indian vessels of wooden construction.

Part IV: Miscellaneous Provisions:

- Carriage of relevant IMO publications: In addition to any other requirement, following publications are required:

(1) Every ship shall be provided with a copy of SOLAS, and MARPOL, as amended; and

(2) a copy each of appropriate codes referred to in these rules as applicable to specific cargo intended to be carried on board.

(3) All major or non-major ports on the coast of India shall at least maintain a copy of following publication namely:

(a) copy of MARPOL 73/78 Convention as amended;

(b) copy of Container Safety Convention 1972 as amended;

(c) copy IMSBC Code as amended;

(d) copy of IMDG Code as amended;

(e) copy each of relevant notice or recommendation of organization, applicable to cargo being handled in such port.

(4) Every other entity including the shipper, manufacture, Laboratory or test houses shall maintain at least a copy of publications specified in Rule 19(3), applicable to nature of trade or services.

- Inspection, survey and certification fee: Fee for departmental surveyors shall be levied as per schedule of fee prescribed by the Director General of Maritime Administration from time to time.

Existing fee structure of subject

| 1 | Grain Stability booklet | (i) Vessels up to 5000 GT (ii) Above 5000 GT | (i) Rs.40,000/- (ii) Rs.40,000/- plus Rs.500/- for every additional 100 GT subject to a maximum of Rs.80,000/- |
| --- | --- | --- | --- |
| 2 | Grain Stability booklet with untrimmed End Calculation. |  | Rs.20,000/- extra |
| 3 | Approval of Grain Fittings |  | Rs.15,000/- extra |
| 4 | Document of authorization |  | Rs.15,000/- extra |
| 5 | Cargo Securing Manual |  | Rs.25,000/- extra |
| 6 | Loading Operation Manuals of Bulk Carriers | (i) Vessels below 150 metres. (ii) Vessels above 150 metres. | (i) Rs.30,000/- (ii) Rs.50,000/- |
| 7 | (i) Inspection of Grain loading Ships - vessels upto 2000 G.T. (ii) Above 2000 G.T. |  | (i) Rs.15,000/- (ii) Rs.15,000/- plus Rs.500/-for every 100 G.T. subject to a maximum of Rs.60,000/-. |
| 8 | Approval and Stamping of additional copies of booklets/plans |  | 10% of approval fees |
| 9 | Endorsement of additional loading conditions / amendments which does not involve re-approval of the plans/booklets |  | 10% of approval fees |

- Penalties:

Whoever contravenes any of the provisions of these rules shall be punishable with fine and imprisonment as specified in the relevant Section of Merchant Shipping Act, 2025.

- Appeal:

The ship owner or his manager or agent has the right to appeal against the decision of the authorized inspecting body. The appeal shall be made to the Director General of Maritime Administration within a period of 7 days from the date of incident. The decision of the Director General shall be final and binding upon the applicant and the authorized inspecting body.

Recommendations for safe carriage for such cargoes issued and amended by the International Maritime Organization from time to time, are to be complied with.

Refer to Form for Cargo information (MSC/Circ.663)

Reference to Documents in this Rule does not preclude the use of electronic data processing (EDP) and electronic date exchange (EDI) transmission techniques as an aid to paper documentation.

The term container is as per the International Convention for Safet Containers (CSC), 1972, as amended, taking into account Guidelines for the Approval of offshore containers handles in open seas (MSC/Circ.860) and Revised recommendations on harmonized interpretation and implementation of the International Convention on Safe Containers, 1972, as amended (CSC.1/Circ.138/Rev.1)

Refer Guidelines Regarding the verified gross mass of a container carrying cargo (MSC.1/Circ.1475)

This document may be presented by means of electronic data processing (EDP) and electronic date exchange (EDI) transmission techniques. The signature may be an electronic signature or may be replaced by the name in capitals, of the person authorized to sign.

Refer to:

.1 Revised recommendations on the safe use of pesticides in ships (MSC.1/Circ.1358);

.2 Recommendations on the safe use of pesticides in ships applicable to the fumigation of cargo holds (MSC.1/Circ.1264, as amended); and

.3 Revised recommendations on the safe use of pesticides in ships applicable to the fumigation of cargo transport units (MSC.1/Circ.1361).

Recommendations on the safe use of pesticides in ships applicable to the fumigation of cargo holds (MSC.1/Circ.1264, as amended)

Refer to the Code of Safe Practice for Cargo Stowage and Securing (resolution A.714(17), as amended).

Refer to the International Maritime Dangerous Goods (IMDG) Code (resolution MSC.122(75), as amended)

Refer to Revised guidelines for the preparation of the Cargo Securing Manual (MSC.1/Circ.1353).

Refer to Recommendations for material safety data sheets (MSDS) for MARPOL Annex / oil cargo and oil fuel (resolution MSC.286(86), as may be amended).

Refer to Guidelines for the transport and handling of limited amounts of hazardous and noxious liquid substances in bulk in offshore support vessels A.673(16), as amended).

Refer to the Code of Practice for the Safe Loading and Unloading of Bulk Carriers (BLU Code) (resolution A.862(20), as amended).

the Code of Safe Practice for Ships Carrying Timber Deck Cargoes, 2011 (2011 TDC Code) (resolution A.1048(27)), Guidance note on precautions to be taken by masters of ships below 100 m in length engaged in carriage of logs (MSC/Circ.525), and Guidance note on precautions to be taken by masters of ships engaged in the carriage of timber cargoes (MSC/Circ.548)