DRAFT
MINISTRY OF PORTS, SHIPPING, AND WATERWAYS
(Shipping Wing)
New Delhi, the _____________,2026
Merchant Shipping (Carriage of Dangerous Goods) Rules, 2026

G.S.R.------- (E)-In exercise of the powers conferred by Section ------  of Merchant Shipping Act, 2025 and in supersession of the Merchant Shipping (Carriage of Cargo) Rules, 1995, except as respects things done or omitted to be done before such supersession, the Central Government hereby makes the following rules namely:

PART I: General Provisions 
1. Short title, commencement: 
(1) These rules may be called Merchant Shipping (Carriage of Dangerous Goods) Rules, 2026.
(2) They shall come into force on the date of their publication in the Official Gazette.
2. Application: unless specified otherwise, these Rules shall apply as follows:
(1) All Indian ships carrying or about to carry cargoes specified in these rules anywhere.
(2) All ships carrying or about to carry cargoes specified in these rules in port or places in India.
(3) Part II – The carriage of dangerous goods in packaged form, is applicable to ships carrying such cargoes. Carriage of dangerous goods in packaged form is prohibited except in accordance with the provisions of this part.

(4) Part III – The carriage of dangerous goods in solid form in bulk, is applicable to ships carrying such cargoes. Carriage of dangerous goods in solid-bulk form is prohibited except in accordance with the provisions of this part.
(5) Part IV – Construction and equipment of ships carrying dangerous liquid chemicals in bulk, is applicable to ships built on or after 01 Jul 1986, and such ships shall comply with the rules of this part, in addition to other applicable rules and regulations.
(6) Part V – Construction and equipment of ships carrying liquified gases in bulk, is applicable to ships built on or after 01 Jul 1986, and such ships shall comply with the rules of this part, in addition to other applicable rules and regulations.
(7) Part VI – Special requirements for the carriage of packaged irradiated nuclear fuel (INF), plutonium and high-level radioactive wastes on board ships, is applicable to to all ships, including less than 500 GT when carrying such cargoes.
3. Exceptions: As provisioned in sub-section (a) of Section 114 of the Merchant Shipping Act, 2025, and unless otherwise specified, these rules shall not apply to the following:
(1) Ship's stores and security equipment as permitted by the Central Government for use of security personnel, in limited quantities; and 
(2) Ship’s salvage equipments for the purpose of salvage services; and
(3) Indian ships of below 500GT, provided competent authority by notification or by specific or general order or notice or circular in writing may decide to the extend or permit other effective measures to ensure reasonable and equivalent safety of such ships taking into account the nature and conditions of sheltered voyage.
(4) Rule 2(7) does not apply to warships, naval auxiliary or other vessels owned or operated by the Central or State Government, and used for the time being only on government non-commercial service
4. Definition: In these rules, unless the context otherwise requires: 
(1) "Act" means the Merchant Shipping Act, 2025.
(2)  The terms "Administration" or "Contracting Government" when used in International Conventions or Codes adopted by the International Maritime Organization, mean the "Central Government" in exercise of powers under the Act administering through Director General of Maritime Administration. 

(3) “Approved” means:
(a) in respect of cargo stowage plans, equipment, packaging, cargo securing manuals, fumigation, identification of hazards of cargoes not listed in the relevant codes and related matters approved by Nautical Advisor to the Government of India or Principal Officer, regional Mercantile Marine Department or Deputy Nautical Advisor to the Government of India or Recognized Organization (RO) notified by the Central Government.
(b) in respect of construction and equipment under MARPOL 73/78, IBC and IGC Codes, means approved by Chief Surveyor with the Government of India or Deputy Chief Surveyor with the Government of India or Recognized Organization (RO) notified by the Central Government.

(c) in respect of stability and strength of ship means approved by Chief Ship Surveyor with the Government of India or Deputy Chief Ship Surveyor with the Government of India or Recognized Organization (RO) notified by the Central Government. 

(4) “Approved person” means any person approved by Rule 4(3) possessing the knowledge and experience required to conduct various tests on machinery, electrical installations, ships hull and structure, equipment, required under these rules in course of construction, major conversions/repairs, operations and maintenance of the ship and appointed under Section 8 of the Act.
(5) "IMSBC Code" means the International Maritime Solid Bulk Cargoes Code, adopted by the International Maritime Organization by Resolution MSC.268(85), as may be amended.
(6) "BLU code" means The Code of Practice for the Safe Loading and Unloading of Bulk Carriers including BLU Manual adopted by the International Maritime Organization by Resolution A.862(20), as amended from time to time; incorporated as Supplement to the IMSBC Code.
(7) "Cargo Unit" means a vehicle, container, flat, pallet portable tank, packaged unit, or any other entity etc., and loading equipment or part thereof which belongs to the ship but is not fixed to the ship.
(8) "Cargo Stowage and Securing Code" or "CSS" means the Code of Safe Practice for Cargo Stowage and Securing adopted by the International Maritime Organization by Resolution A.714 (16), as amended. 

(9) "CTU Code" means the 2014 Code of Practice for Packing of Cargo Transport Units, which is endorsed by IMO, ILO, and UNECE, and is as amended from time to time.
(10) “Cargo Transport Unit” is a freight container, swap body, vehicle, railway wagon or any other similar unit in particular when used in intermodal transport.
(11) "Chemical tanker" means a cargo ship constructed or adopted and used for the carriage in bulk any liquid product listed in Chapter 17 of the International Bulk Chemical (IBC) Code.

(12) “Combination carrier” means a ship designed to carry either oil or solid cargoes in bulk.
(13) “Competent Authority” means Nautical Adviser to the Government of India, for the purpose of these rules to supplement the provisions to ensure that appropriate information on cargo, its stowage and securing is provided along with specific precautions, where necessary, for the safe carriage of such cargoes.
(14) “Container Safety Convention” (CSC) means the Convention on Safe Containers,1972, adopted by the International Maritime Organization, as amended from time to time.       
(15) “Director General” means the Director General of Maritime Administration appointed under Section 7(1) of the Act for the purpose of implementing and enforcing the provisions of these rules.
(16) "Dangerous Goods" means dangerous cargoes carried in packaged form or solid form in bulk, and includes harmful substances identified as marine pollutants in the International Maritime Dangerous Goods (IMDG) Code. 
(17) “Dangerous goods in solid form in bulk” means any material, other than liquid or gas, consisting of a combination of particles, granules or any larger pieces of material, generally uniform in composition, which is covered by the IMDG Code and is loaded directly into the cargo spaces of a ship without any intermediate form of containment, and includes such materials loaded in a barge on a barge-carrying ship.
(18) "Gas carrier" means a cargo ship constructed or adopted and used for the carriage in bulk of any liquefied gas or other product listed in Chapter 19 of the International Gas Carrier (IGC) Code.

(19) "International Bulk Chemical (IBC) Code" means the International Code for the Construction and Equipment of Ships Carrying Dangerous Chemicals in Bulk, adopted by the International Maritime Organization, as amended form time to time, applicable to ships built on or after 1st July, 1986. 

(20) "International Gas Carrier (IGC) Code" means the International Code for the Construction and Equipment of Ships Carrying Liquefied Gases in Bulk, adopted by the International Maritime Organization, as amended from time to time, applicable to ships built on or after 1st July, 1986. 

(21) "International Maritime Dangerous Goods Code" or "IMDG code" means the International Maritime Dangerous Goods code adopted by the International Maritime Organization by Resolution A.716(17), as amended from time to time.

(22) "INF Code" means International Code for the Safe Carriage of Packaged Irradiated Nuclear Fuel, Plutonium and high-level radioactive wastes on board ships, adopted by the International Maritime Organization, by Resolution MSC.88(71), as may be amended from time to time.

(23) “INF Cargo” means packaged irradiated nuclear fuel, plutonium and high-level radioactive wastes carried as cargo in accordance with Class 7 of the IMDG Code.
(24) “Irradiated nuclear fuel” means material containing uranium, thorium and/or plutonium isotopes which has been used to maintain a self-sustaining nuclear chain reaction.

(25) "MARPOL 73/78 Convention" means the International Convention for Prevention of Pollution from Ships, in force, including its protocol adopted by the International Maritime Organization, as amended from time to time. 

(26) Term "notice" includes a Merchant Shipping (MS) Notice, Directorate General of Maritime Administration Order or Departmental Circular, issued from time to time.
(27) "Organization" means the International Maritime Organization (IMO) – a United Nation's specialized agency with responsibility for the safety and security of shipping and the prevention of marine pollution by ships.
(28) “Oil tanker” means a ship designed and constructed to carry oil in bulk in its cargo spaces and includes combination carrier, NLS tanker as defined in Annex II of MARPOL, and any gas carrier as defined in SOLAS Regulation II-1/3.20, as amended when carrying a cargo or part cargo of oil in bulk.

(29) “Packaged Form” means the form of containment specified in the IMDG Code
(30) “Plutonium” means he resultant mixture of isotopes of that material extracted from irradiated nuclear fuel from reprocessing.

(31) “High-level radioactive wastes” means liquid wastes resulting from the operation of the first stage extraction system or the concentrated wastes from subsequent extraction stages, in a facility for reprocessing irradiated nuclear fuel, or solids into which such liquid wastes have been converted.

(32) "Recommendation of the Organization" or "Recommended by Organization" includes its assembly or its committee resolution or its circular, as adopted by the Organization. 
(33) "SOLAS Convention" or "SOLAS" means the International Convention for Safety of Life at Sea, 1974, in force including its protocols adopted by the International Maritime Organization and as amended from time to time.
(34) “Solid Bulk Cargo” means any cargo, other than liquid or gas, consisting of a combination of particles, granules or any larger pieces of material generally uniform in composition, which is loaded directly into the cargo spaces of a ship without any intermediate form of containment.
(35) “Terminal Representative” means a person appointed by the terminal or other facility where the ship is loading or unloading, who has responsibility for operations conducted by that terminal or facility with regard to the particular ship.
(36) "U.N. number" means a serial number assigned to a dangerous substance in the International Maritime Dangerous Goods (IMDG) Code.
(37) Meaning of other terms and abbreviations: All other terms and abbreviations which are used in these rules and defined in the Act, SOLAS 1974, MARPOL 73/78, CSC 1972 and applicable Codes for the purpose of safe carriage of cargoes, cargo units and cargo transport units shall have the meanings as defined in the Act and those applicable Regulations, Codes and Conventions. 
PART II: Carriage of dangerous goods in packaged form 
5. Requirements for the carriage: 
(1) The carriage of dangerous goods in packaged form shall be in compliance with the relevant provisions of the IMDG Code.
(2) Ships carrying dangerous goods shall comply with special requirements as given in SOLAS Regulation II-2/19, Recognized Organizations upon ascertaining compliance, shall issue document of compliance as per format in ‘First Schedule’
(3) to supplement the provisions of this part, Directorate General of Maritime Administration may issue detailed instructions vide ‘notice’ on emergency response
 and medical first aid
 relevant to incidents involving dangerous goods in packaged form.
6. Documents
: In addition to any other documents required as per other applicable rules and regulations, following documents are required:
(1) Transport information relating to carriage of dangerous goods in packaged form and the container/vehicle packing certificate shall be in accordance with relevant provisions of the IMDG Code and a certified copy of same shall be made available to the nearest authority of the port State. For this purpose, on the coast of India, the nearest Mercantile Marine Department (MMD) shall be treated as Port State authority.

(2) Such ship shall have a special list, manifest or stowage plan setting forth, in accordance with the relevant provisions of the IMDG Code, the dangerous goods on board and the location thereof. A certified copy of one of these documents shall be made available to the nearest authority of the port State. For this purpose, on the coast of India, the nearest Mercantile Marine Department (MMD) shall be treated as Port State authority.
(3) In compliance with SOLAS Chapter VII and MARPOL Annex III, dangerous goods transported by ships, shall comply with documentation as required by Chapter 5.4 of the IMDG Code, as amended 
7. Cargo securing, stowage and segregation requirements:
(1) Shipper and Master of the vessel loading dangerous goods in packaged form is to ensure that the dangerous goods being loaded on ship is packed, labeled, and classified, as per the applicable provisions of the IMDG Code, as amended.

(2) Each ship carrying dangerous goods in packaged form consisting of cargo units and cargo transport units, shall be loaded and stowed in compliance with the applicable provisions of the IMDG Code, as amended.
(3) Each ship carrying dangerous goods in packaged form consisting of cargo units and cargo transport units, shall be secured as per the approved cargo securing manual
.

(4) Packing of each IMDG cargo unit including intermediate bulk containers (IBC) and large packings, shall be certified by an entity / organization approved by the Directorate General of Maritime Administration, to ensure that the said packing conforms to design type successfully tested, and meets the requirements for the construction, testing, and approval of the packages to be used for carriage of IMDG cargo, in accordance with the IMDG Code.
(5) To ensure compliance with national and international requirements pertaining to the carriage of IMDG cargo, all vessels loading IMDG cargo in Indian ports shall submit stowage plan sufficiently in advance, to the jurisdictional MMDs. Surveyors of the MMDs may inspect such vessels to ensure compliance with the relevant applicable requirements, however, with regard to carriage of Class 1 & Class 7 IMDG cargo, the jurisdictional MMD’s Surveyor shall inspect the stowage aspects on the vessel and issue NOC for the vessel to sail out of the port with such cargo.
(6) Explosives under IMDG Class 1, Division 1.1 shall be loaded or discharged only at place(s) or port(s) designated and approved by the Controller of Explosives under the Explosives Act 1884, as amended, or any of the Armed Forces of the Union of India, and Ordnance Factories or other establishments of such Forces in accordance with Rules made by the Central Government, from time to time.

(7) Radioactive materials under IMDG Class 7 shall be loaded or discharged only at place(s) or ports(s) designated, approved and supervised by the competent authority under the Atomic Energy Act 1962, as amended.
(8) Marine pollutant and wastes: many of the substances assigned to Classes 1 to 9 are deemed to be as marine pollutant as classified in IMDG Code, such substances carriage shall not only comply with the applicable provisions of the IMDG Code but also to that of MARPOL Annex III. Wastes shall be transported under the provisions of the appropriate class, considering their hazards and the criteria in the Code.  Wastes not otherwise subject to the Code but covered under the UN's BASEL Convention, 1989, may be transported under Class 9 and comply with applicable provisions of the IMDG Code.
8. Reporting:

(1) When an incident takes place involving the loss or likely loss overboard of dangerous goods in packaged form into the sea, the master, or other person having charge of the ship, shall report the particulars of such an incident without delay and to the fullest extent possible to the DGCOMM Centre, and in case of Indian vessel in foreign waters then should also report to the nearest coastal state. The report shall be drawn up based on guidelines developed by the Organization
.
(2) In the event of the ship referred to in Rule 8(1) being abandoned, or in the event of a report from such a ship being incomplete or unobtainable, the Company, as defined in regulation SOLAS Regulation IX/1.2, shall, to the fullest extent possible, assume the obligations placed upon the master by this Rule.
PART III: Carriage of dangerous goods in solid bulk form 
9. Requirements for the carriage: 

(1) The carriage of ‘dangerous goods in solid bulk’ shall be in compliance with the relevant provisions of the IMDG Code, and the IMSBC Code.
(2) Ships carrying dangerous goods shall comply with special requirements as given in SOLAS Regulation II-2/19, Recognized Organizations upon ascertaining compliance, shall issue document of compliance as per format in ‘First Schedule’
(3) to supplement the provisions of this part, Directorate General of Maritime Administration may issue detailed instructions vide ‘notice’ on emergency response
 and medical first aid
 relevant to incidents involving dangerous goods in packaged form.
10. Documents:

(1) In all documents relating to the carriage of dangerous goods in solid form by sea, the bulk cargo shipping name of the goods shall be used (trade names alone shall not be used).
(2) Ships carrying dangerous goods in sold bulk form shall have a special list or manifest setting forth the dangerous goods on board and the location thereof. A detailed stowage plan, which identifies by class and sets out the location of all dangerous goods on board, may be used in place of such a special list or manifest. A certified copy of one of these documents shall be made available before departure to the nearest authority of the port State. For this purpose, on the coast of India, the nearest Mercantile Marine Department (MMD) shall be treated as port state authority.

11. Stowage and segregation:
(1) Be loaded and stowed safely and appropriately in accordance with the nature of the goods. Incompatible goods shall be segregated from one another.

(2) Such cargo if liable to spontaneous heating or combustion, shall not be carried unless adequate precautions have been taken to minimize the likelihood of the outbreak of fire. In any such case, if loading from a port in India, necessary permission for loading needs to be taken from jurisdictional MMD. 

(3) Such cargoes, which gives off dangerous vapors, shall be stowed in a well-ventilated cargo space.
12. Reporting:

(1) When an incident takes place involving the loss or likely loss overboard of dangerous goods in packaged form into the sea, the master, or other person having charge of the ship, shall report the particulars of such an incident without delay and to the fullest extent possible to the DGCOMM Centre and in case of Indian vessel in foreign waters then should also report to the nearest coastal state. The report shall be drawn up based on guidelines developed by the Organization
.
(2) In the event of the ship referred to in Rule 8(1) being abandoned, or in the event of a report from such a ship being incomplete or unobtainable, the company, as defined in regulation SOLAS Regulation IX/1.2, shall, to the fullest extent possible, assume the obligations placed upon the master by this Rule.
PART IV: Construction and equipment of ships carrying dangerous liquid chemicals in bulk:
13. Requirements for Chemical Tanker:
(1) A chemical tanker as defined in Rule 4(11) shall comply with requirements of the International Bulk Chemical (IBC) Code, and be certified accordingly.

(2) In addition to Rule 13(1), such chemical tanker shall also comply with applicable requirements of the SOLAS Regulation I/8 (Survey of life saving appliances and other equipment of cargo ships
), SOLAS Regulation I/9 (Surveys of radio installations of cargo ships
), and SOLAS I/10 (Surveys of structure, machinery and equipment of cargo ships
) and be surveyed and certified accordingly.
(3) A chemical tanker certified as per Rule 13(1) and 13(2), shall be subject to Control established as per SOLAS Regulation I/19 (Procedures for Port State Control
). For this purpose such certificate shall be treated as a certificate issued under SOLAS Regulation I/12 (Issue or endorsement of certificates
) or I/13 (Issue or endorsement of certificates by another Government)
PART V: Construction and equipment of ships carrying liquified gases in bulk:
14. Requirements for Gas Carriers:

(1) A gas carrier as defined in Rule 4(18) shall comply with requirements of the International Gas Carrier (IGC) Code, and be certified accordingly.

(2) In addition to Rule 14(1), such gas carrier shall also comply with applicable requirements of the SOLAS Regulation I/8 (Survey of life saving appliances and other equipment of cargo ships
), SOLAS Regulation I/9 (Surveys of radio installations of cargo ships
), and SOLAS I/10 (Surveys of structure, machinery and equipment of cargo ships
) and be surveyed and certified accordingly. For this purpose such certificate shall be treated as a certificate issued under SOLAS Regulation I/12 (Issue or endorsement of certificates
) or I/13 (Issue or endorsement of certificates by another Government)
(3) A gas carrier certified as per Rule 14(1) and 14(2), shall be subject to Control established as per SOLAS Regulation I/19 (Procedures for Port State Control
).

PART VI: Special requirements for carriage of packaged irradiated nuclear fuel, plutonium and high-level radioactive waste on board ships:
15. Requirements for ships carrying INF cargo:
(1) A ship carrying INF cargo as defined in Rule 4(23) shall comply with requirements of the INF Code, and be certified accordingly.

(2) In addition to Rule 15(1), such vessel such ship shall also comply with all other applicable requirements, and be surveyed and certified accordingly.

(3) A ship certified as per Rule 15(1) and 15(2), shall be subject to Control established as per SOLAS Regulation I/19 (Procedures for Port State Control
). For this purpose such certificate shall be treated as a certificate issued under SOLAS Regulation I/12 (Issue or endorsement of certificates
) or I/13 (Issue or endorsement of certificates by another Government)
Part VII: Miscellaneous Provisions:
16. Carriage of cargo on vessels registered under Part XIII of the Act: 

(1) An Indian vessel registered under provisions of Part XIII of the Act, anywhere and any other non-conventional vessel of wooden construction, when in Indian waters shall not carry following cargoes- 
a) Any cargo in bulk, which comes in direct contact with her wooden hull and does not allow access to repair/attend to wooden hull, as may be necessary in need; or

b) Dangerous cargoes. 

(2) For purpose of promoting coastal trade, in India and with neighboring littoral countries and for providing such balance of trade between neighboring littoral countries or for supporting any other Government of India policy/initiative, the Director General may prescribe carriage of certain type of cargoes on board Indian vessels of wooden construction.

17. Carriage of relevant IMO publications: In addition to any other requirement, following publications are required:
(1) Every ship shall be provided with a copy of SOLAS, and MARPOL, as amended; and  
(2) a copy each of appropriate codes referred to in these rules as applicable to specific cargo intended to be carried on board.  
(3) All major or non-major ports on the coast of India shall at least maintain a copy of following publication namely:
(a) copy of MARPOL 73/78 Convention as amended;

(b) copy of Container Safety Convention 1972 as amended;

(c) copy IMSBC Code as amended; 

(d) copy of IMDG Code as amended;

(e) copy each of relevant notice or recommendation of organization, applicable to cargo being handled in such port.

(4) Every other entity including the shipper, manufacture, Laboratory or test houses shall maintain at least a copy of publications specified in Rule 19(3), applicable to nature of trade or services.

18. Inspection, survey and certification fee: Fee for departmental surveyors shall be levied as per schedule of fee prescribed by the Director General of Maritime Administration from time to time. 
19. Penalties:

Whoever contravenes any of the provisions of these rules shall be punishable with fine and imprisonment as specified in the relevant Section of Merchant Shipping Act, 2025.

20. Appeal:

The ship owner or his manager or agent has the right to appeal against the decision of the authorized inspecting body. The appeal shall be made to the Director General of Maritime Administration within a period of 7 days from the date of incident. The decision of the Director General shall be final and binding upon the applicant and the authorized inspecting body.
� Refer to the Revised Emergency Response Procedures for ships carrying dangerous goods (EmS Guide) (MSC.1/Circ.1588)


� Refer to the Medical First Aid Guide for use in accidents involving dangerous goods (MFAG) which is reproduced in the Supplement of the IMDG Code. published by the Organization.


� The Reference to Documents in this Rule does not preclude the use of electronic data processing (EDP) and electronic date exchange (EDI) transmission techniques as an alternate to paper documentation, and includes use of EDP and EDI as transmission techniques, as detailed in IMDG Code Chapter 5.4


� Refer to Revised guidelines for the preparation of the Cargo Securing Manual (MSC/Circ.1353).


� Refer to General principles for ship reporting systems and ship reporting requirements, including guidelines for reporting incident involving dangerous goods, harmful substances and/or marine pollutants (resolution A.851(20), as amended)


� Refer to the Revised Emergency Response Procedures for ships carrying dangerous goods (EmS Guide) (MSC.1/Circ.1588)


� Refer to the Medical First Aid Guide for use in accidents involving dangerous goods (MFAG) which is reproduced in the Supplement of the IMDG Code. published by the Organization.


� Refer to General principles for ship reporting systems and ship reporting requirements, including guidelines for reporting incident involving dangerous goods, harmful substances and/or marine pollutants (resolution A.851(20), as amended)


� Refer to Guidelines for construction, installation, maintenance and inspection/survey of means of embarkation and disembarkation (MSC.1/Circ.1331); Servicing of life-saving appliances and radiocommunication equipment under the Harmonized System of Survey and Certification (HSSC) (MSC/Circ.955); Guidelines for the performance and testing criteria and surveys of high-expansion foam concentrates for fixed-extinguishing systems (MSC/Circ.670); Guidelines for performance and testing criteria and surveys of medium expansion from concentrates for fixed fire-extinguishing systems (MSC/Circ.798) and Guidance on the survey and certification of compliance of ships with the requirement to transmit LRIT information (MSC.1/Circ.1307).


� Refer to Servicing of life-saving appliances and radiocommunication equipment under the Harmonized System of Survey and Certification (HSSC) (MSC/Circ.955);


� Refer to Guidance for planning the enhanced programme of inspections during surveys of bulk carriers and oil tankers (MSC/Circ.655); Guidelines for pre-planning of surveys in dry-dock of ships which are not subject to the Enhanced programme of inspections (MSC.1/Circ.1223);


� Refer to Procedures for port State control, 2019 (resolution A.1138(31)); IMO Instruments Implementation Code (III Code) resolution A.1070(28)); Guidance to port State control officers on the non-security related elements of the 2002 SOLAS amendments (MSC/Circ.1113); Measures to improve port State control procedures (MSC/Circ.11011-MEPC/Circ.383); Code of good practice for port State control officers (MSC-MÉPC.4/Circ.2); National contact points for safety and pollution prevention and response (MSC MEPC.6/Circ.18, as may be amended), and the information collected via the Global Integrated Shipping Information System (GISIS)


� Refer to Recommendation on the use o f national tonnage i n applying international conventions (resolution A.1073(28)); Guidelines for the use of Electronic Certificates (FAL.5/Circ.39/Rev.2); Guidance on the timing of replacement of existing certificates by the certificates issued after the entry into force of amendments to certificates in IMO instruments (MSC-MEPC.5/Circ.6); Unified interpretation on the


expiration date of statutory certificates (MSC-MEPC.5/Circ.13) and Unified interpretation of the date of completion of the survey and verification on which the certificates are based (MSC-MEPC.5/Circ.3).


� Refer to Guidelines for construction, installation, maintenance and inspection/survey of means of embarkation and disembarkation (MSC.1/Circ.1331); Servicing of life-saving appliances and radiocommunication equipment under the Harmonized System of Survey and Certification (HSSC) (MSC/Circ.955); Guidelines for the performance and testing criteria and surveys of high-expansion foam concentrates for fixed-extinguishing systems (MSC/Circ.670); Guidelines for performance and testing criteria and surveys of medium expansion from concentrates for fixed fire-extinguishing systems (MSC/Circ.798) and Guidance on the survey and certification of compliance of ships with the requirement to transmit LRIT information (MSC.1/Circ.1307).


� Refer to Servicing of life-saving appliances and radiocommunication equipment under the Harmonized System of Survey and Certification (HSSC) (MSC/Circ.955);


� Refer to Guidance for planning the enhanced programme of inspections during surveys of bulk carriers and oil tankers (MSC/Circ.655); Guidelines for pre-planning of surveys in dry-dock of ships which are not subject to the Enhanced programme of inspections (MSC.1/Circ.1223);


� Refer to Recommendation on the use o f national tonnage i n applying international conventions (resolution A.1073(28)); Guidelines for the use of Electronic Certificates (FAL.5/Circ.39/Rev.2); Guidance on the timing of replacement of existing certificates by the certificates issued after the entry into force of amendments to certificates in IMO instruments (MSC-MEPC.5/Circ.6); Unified interpretation on the


expiration date of statutory certificates (MSC-MEPC.5/Circ.13) and Unified interpretation of the date of completion of the survey and verification on which the certificates are based (MSC-MEPC.5/Circ.3).


� Refer to Procedures for port State control, 2019 (resolution A.1138(31)); IMO Instruments Implementation Code (III Code) resolution A.1070(28)); Guidance to port State control officers on the non-security related elements of the 2002 SOLAS amendments (MSC/Circ.1113); Measures to improve port State control procedures (MSC/Circ.11011-MEPC/Circ.383); Code of good practice for port State control officers (MSC-MÉPC.4/Circ.2); National contact points for safety and pollution prevention and response (MSC MEPC.6/Circ.18, as may be amended), and the information collected via the Global Integrated Shipping Information System (GISIS)


� Refer to Procedures for port State control, 2019 (resolution A.1138(31)); IMO Instruments Implementation Code (III Code) resolution A.1070(28)); Guidance to port State control officers on the non-security related elements of the 2002 SOLAS amendments (MSC/Circ.1113); Measures to improve port State control procedures (MSC/Circ.11011-MEPC/Circ.383); Code of good practice for port State control officers (MSC-MÉPC.4/Circ.2); National contact points for safety and pollution prevention and response (MSC MEPC.6/Circ.18, as may be amended), and the information collected via the Global Integrated Shipping Information System (GISIS)


� Refer to Recommendation on the use o f national tonnage i n applying international conventions (resolution A.1073(28)); Guidelines for the use of Electronic Certificates (FAL.5/Circ.39/Rev.2); Guidance on the timing of replacement of existing certificates by the certificates issued after the entry into force of amendments to certificates in IMO instruments (MSC-MEPC.5/Circ.6); Unified interpretation on the


expiration date of statutory certificates (MSC-MEPC.5/Circ.13) and Unified interpretation of the date of completion of the survey and verification on which the certificates are based (MSC-MEPC.5/Circ.3).





21

